Aim: - To write C++ code of the given following tasks.

Software Used: -

Cloud Keil Studio IDE and Tera Term

Theory: -

The mbed NXP LPC11U24 Microcontroller in particular is designed for prototyping low-cost USB devices, battery powered applications and 32-bit ARM® Cortex™-M0 based designs. It is packaged as a small DIP form-factor for prototyping with through-hole PCBs, stripboard and breadboard, and includes a built-in USB FLASH programmer.

Specifications of LPC11U24:

- NXP LPC11U24 MCU

- Low power ARM® Cortex™-M0 Core

- 48MHz, 8KB RAM, 32KB FLASH

- USB Device, 2xSPI, I2C, UART, 6xADC, GPIO

- Prototyping form-factor

- 40-pin 0.1" pitch DIP package, 54x26mm

- 5V USB, 4.5-9V supply or 2.4-3.3V battery

- Built-in USB drag 'n' drop FLASH programmer

- mbed.org Developer Website

- Lightweight Online Compiler

- High level C/C++ SDK

- Cookbook of published libraries and projects

API Used:

- Syntax used for digital output:

- DigitalOut (PinName pin)

- For LED blinking we’ve used:

- DigitalOut variable(LEDn); where n= 1,2,3,4

- For delay:

- wait(t); where ‘t’ is in seconds

Task 1: - Write a C++ code with mbed  APIS to generate a PWD signal with duty cycle 50%. Verify using buzzer/led and digital oscilloscope.

Code: -

#include "mbed.h"

PwmOut led(PC_8);

int main()

{

//specify period first

led.period(1.0f);

led.write(0.2f); //set duty cycle, relative t period

//led = 0.5f;      //shorthand for led.write();

//led.pulsewidth_ms(50);

while(1); //used to generate continous waveform

}

Output: -

Task 2: - Write a C++ code with mbed APIs generate a PWM signal with duty cycle 20%. Verify the output using LED, Digital oscilloscope and PC Monitor.

Code: -

#include "mbed.h"

#include "mbed2/299/drivers/PwmOut.h"

PwmOut led(PC_8);

Serial pc(USBTX,USBRX);

int main()

{

led.period(1.0f);

led.write(0.2f);

while(1)

{

pc.printf("High");

wait(0.2);

pc.printf("Low");

wait(0.8);

}

}

Output: -

Task 3: - Controlling the Led brightness by using POT on the analog pin while the PWM period remains the same.

Code: -

#include "mbed.h"

PwmOut PWM1(PC_8);

AnalogIn Ain(PC_5);

int main()

{

while(1)

{

PWM1.period(0.5);

//set PWM period to 10ms

PWM1 = Ain;

wait(0.1);

}

}

Output: -

Task 4: - Generate a PWM signal to increase the brightness of LED when pressed “I” and then decrease the brightness when pressed “d”.

Code: -

#include "mbed.h"

PwmOut PWM1(PC_8);

PwmOut led(PC_8);

Serial pc(USBTX, USBRX);

DigitalOut myled(LED1);

int main()

{

float brightness = 0.0f;

while(1)

{

pc.printf("Press i(to increase) and d(to decrease) the brightness of LED: ");

char y = pc.getc();

if(y == 'd'){

brightness -= 0.1f;

}

else if(y == 'i'){

brightness += 0.1f;

}

else{

pc.printf("Please enter a valid character!");

}

led.write(brightness);

wait(0.1);

}

}

Output: -

Verification Status: -

Result: -

Successfully understood and performed all the given tasks.