Aim: - To write C++ source code to understand the digital port access.

Software Used: -

Cloud Keil Studio IDE and Tera Term

Theory: -

The mbed NXP LPC11U24 Microcontroller in particular is designed for prototyping low-cost USB devices, battery powered applications and 32-bit ARM® Cortex™-M0 based designs. It is packaged as a small DIP form-factor for prototyping with through-hole PCBs, stripboard and breadboard, and includes a built-in USB FLASH programmer.

Specifications of LPC11U24:

- NXP LPC11U24 MCU

- Low power ARM® Cortex™-M0 Core

- 48MHz, 8KB RAM, 32KB FLASH

- USB Device, 2xSPI, I2C, UART, 6xADC, GPIO

- Prototyping form-factor

- 40-pin 0.1" pitch DIP package, 54x26mm

- 5V USB, 4.5-9V supply or 2.4-3.3V battery

- Built-in USB drag 'n' drop FLASH programmer

- mbed.org Developer Website

- Lightweight Online Compiler

- High level C/C++ SDK

- Cookbook of published libraries and projects

API Used:

- Syntax used for digital output:

- DigitalOut (PinName pin)

- For LED blinking we’ve used:

- DigitalOut variable(LEDn); where n= 1,2,3,4

- For delay:

- wait(t); where ‘t’ is in seconds

Task 1: - Write a c++ code to transmit a message “VIT SENSE” using the STM port serial to pc for every 1 second.

Code: -

#include "mbed.h"

Serial pc(USBTX, USBRX);

int main()

{

while(1)

{

pc.printf("20BLC1087\n");

wait(1);

}

}

Output: -

Task 2: - Write a C++ code with mbed APIs to receive a character(s) from PC and Switch ON the on-board LED. For all other character LED must be in OFF state.

Code: -

#include "mbed.h"

Serial pc(USBTX, USBRX);

DigitalOut myled(LED1);

int main()

{

pc.printf("Press a character");

while (1)

{

char y = pc.getc();

if (y=='s')

{

myled=1;

wait(1);

}

else

{

myled=0;

wait(1);

}

}

}

Output: -

Task 3: -

Write a C++ code with mbed APIs to interface a serial port, switch (PB_13) and LED (PC_4). Perform the below operation:

When switch=1, make LED ON and sent a message “LED ON” to PC serial monitor.
When switch=0, make LED OFF and sent a message “LED OFF” to PC serial monitor.

Code: -

#include "mbed.h"

Serial pc(USBTX, USBRX);

BusIn inps (PB_13);

DigitalOut myled(PC_4);

int main()

{

if (inps == 0b01)

{

myled=1;

pc.printf("LED ON");

wait(1);

}

else if (inps == 0b00)

{

myled=0;

pc.printf("LED OFF");

wait(1);

}

}

Output: -

Verification Status

Result: - Therefore all the above tasks have been successfully understood, experimented and verified successfully.